Hybrid Digital Twin for Real-Time Seismic Response Prediction of Reinforced Concrete Buildings

Miguel Rivera Ospina

Department of Civil Engineering

* Corresponding Author: [author email]

© 2026 Authors. Open Access under CC BY 4.0.

# Title

Hybrid Digital Twin for Real-Time Seismic Response Prediction of Reinforced Concrete Buildings

# Authors

Miguel Rivera Ospina​1,*

​1 Department of Civil Engineering, [YOUR UNIVERSITY], [CITY, COUNTRY]

​*Corresponding Author: [YOUR EMAIL]

Copyright © 2026 by authors, all rights reserved. Authors agree that this article remains permanently open access under the terms of the Creative Commons Attribution License 4.0 International License.

# Abstract

Seismic risk assessment of reinforced concrete (RC) buildings requires thousands of Nonlinear Time History Analyses (NLTHA), prohibitive for real-time monitoring. This paper presents a Hybrid Digital Twin combining OpenSeesPy fiber-section NLTHA with a Physics-Guided Neural Network (PgNN) surrogate for real-time inter-story drift ratio (IDR) prediction. Nonlinear restoring forces fint, floor accelerations, and velocities from OpenSeesPy fiber sections inform per-story inverse-variance weights and physics tensor regularization. The framework scales from N=3 to N=10 stories without architectural modification. Validated on the complete PEER NGA-West2 database, the PgNN achieves R2=0.783 (RMSE =0.834%) for N=3 (289 records) and R2=0.713 (RMSE =0.523%) for N=10 (265 records). The monotonic per-story accuracy degradation from R2=0.754 (base) to R2=0.601 (roof), driven by higher-mode participation (whiplash effect), is systematically characterized. Transfer learning from N=3 retains 98.2% accuracy for N=10, confirming domain-invariant feature extraction. PgNN-derived fragility curves identify mid-height drift concentration as the governing failure mode. CPU inference latency of $$2 ms enables deployment without GPU hardware.

Keywords Hybrid Digital Twin; Physics-Guided Neural Networks; FEM-Guided Surrogate Modeling; Seismic Response Prediction; Inter-Story Drift Ratio; Nonlinear Time History Analysis; Reinforced Concrete Frames; Whiplash Effect

# 1. Introduction

Reinforced concrete (RC) buildings represent a significant portion of the global building stock in seismically active regions. Under moderate to strong ground shaking, these structures exhibit highly nonlinear behavior driven by concrete cracking, steel yielding, and stiffness degradation — phenomena that only Nonlinear Time History Analysis (NLTHA) can rigorously capture using high-fidelity Finite Element Models (FEM) [1]. However, a single NLTHA of a multi-story RC frame requires 10–60 seconds of wall-clock time, making it computationally infeasible for Probabilistic Seismic Demand Analysis (PSDA), which typically requires thousands of simulations to characterize structural fragility across the full hazard spectrum.

Surrogate modeling — the replacement of expensive simulations with computationally efficient approximations — has emerged as a practical solution to this bottleneck. Classical metamodeling approaches, including polynomial response surfaces, Kriging, and polynomial chaos expansions, have demonstrated success in scalar engineering demand parameter (EDP) prediction. However, these methods require explicit feature engineering, scale poorly to multi-dimensional output spaces (e.g., full displacement time-histories), and are inherently non-parametric: each surrogate is fixed to a specific building geometry and must be retrained from scratch for any change in height, plan configuration, or structural system.

Deep learning surrogates have recently addressed these limitations [27]. Convolutional neural networks (CNNs) [21] and long short-term memory networks (LSTMs) have been applied to predict seismic responses from ground motion inputs, achieving R2>0.85 for peak IDR prediction without hand-crafted features [10], while machine learning classifiers have shown promise for failure mode recognition in RC members [30]. Yet, purely data-driven models lack physical interpretability: they can generate dynamically inconsistent predictions — for example, non-zero pre-event displacements or accelerations that violate Newton’s second law — and typically require thousands of FEM simulations to generalize reliably across intensity levels and site conditions.

Physics-Informed and Physics-Guided Neural Networks, introduced by Raissi et al. [3], resolve this trade-off by incorporating governing physical equations into the training process — either through the loss function or through physics-derived training data — constraining predictions to physically consistent solutions. Applied to structural dynamics, the equation of motion

Mu+Cu+fint(u)=−Mιug

(1)

provides a powerful regularization signal that reduces the labeled data requirement and improves generalization to unseen ground motions. Zhang et al. [10] demonstrated that physics-informed multi-LSTM networks can predict nonlinear dynamic responses with fewer training samples than purely data-driven counterparts. However, prior work has approximated the internal force term fint analytically (e.g., via linear stiffness or simplified hysteretic rules), failing to capture the full nonlinear cyclic degradation encoded in FEM fiber sections [7, 8].

Concurrently, the Digital Twin paradigm — a synchronized virtual replica of a physical system updated in real-time from sensor data [4, 6] — provides the operational context for a physics-guided structural surrogate. For seismic applications, the digital twin must produce accurate structural state estimates at latencies compatible with control loop requirements (≤100 ms for semi-active damper actuation), a constraint that full NLTHA cannot satisfy.

Research Gap. Existing surrogate models for seismic response prediction are limited by three concurrent constraints: (1) they are black-box, lacking adherence to laws of motion; (2) they are scalar-only, predicting peak quantities rather than the full displacement time-histories required for damage state classification; and (3) they are non-parametric, fixed to a specific building geometry and unable to generalize to different story counts without retraining. Furthermore, no prior work has validated a physics-informed surrogate against the complete PEER NGA-West2 database at multiple building heights, nor characterized the whiplash effect — progressive upper-story accuracy degradation due to higher-mode participation — in the PgNN context.

Contribution. This paper presents a Hybrid Digital Twin with the following specific contributions:

- A fully parametric OpenSeesPy NLTHA framework scalable from N=3 to N=10+ stories, with fiber-section nonlinearity (Concrete02 [7], Steel02 [8]) and a five-strategy convergence cascade achieving 98.0% record convergence for N=3 and 88.6% for N=10 on the complete PEER NGA-West2 database [2].

- A FEM-Guided Loss with Physics Tensors: actual nonlinear restoring forces fint, floor accelerations u, and velocities u from OpenSeesPy fiber sections — encoding concrete cracking, steel yielding, and cyclic stiffness degradation — inform the training objective through per-story inverse-variance weighting derived from FEM response statistics, replacing the linearized approximations of prior work [10].

- Validated performance on the complete PEER NGA-West2 database: R2=0.783, RMSE =0.834% for N=3 (289 records); R2=0.713, RMSE =0.523% for N=10 (265 records). The monotonic per-story accuracy degradation from R2=0.754 (base) to R2=0.601 (roof) — attributable to higher-mode participation — is characterized in the PgNN context.

- Real-time CPU inference at $$2 ms per prediction, enabling deployment in structural health monitoring and semi-active control loops without GPU hardware.

The overall framework is illustrated (Figure 1). The remainder of this paper is organized as follows. Section 2 states the research objectives. Section 3 describes the parametric FEM model, ground motion database, PgNN architecture, and training protocol. Section 4 presents simulation results and prediction accuracy for both building configurations. Section 5 discusses the whiplash effect, limitations, and practical implications. Section 6 concludes with key findings and future research directions.

Hybrid Digital Twin framework overview

Hybrid Digital Twin framework overview

Figure 1. Hybrid Digital Twin framework: simulation layer (top) generates physics tensors via OpenSeesPy NLTHA; intelligence layer (bottom) trains the PgNN surrogate with FEM-Guided Loss using actual FEM restoring forces fint, floor accelerations, and velocities as physics-guided training signal. Dashed arrow indicates the physics-data coupling between both layers.

# 2. Objectives

The overarching goal of this work is to develop and validate a Hybrid Digital Twin that couples high-fidelity NLTHA with a PgNN surrogate for real-time seismic damage prediction. Specifically, the study aims to:

- Construct a parametric OpenSeesPy RC frame model that scales from N=3 to N=10 stories without manual reconfiguration.

- Train a PgNN using FEM-derived physics tensors as training signal, targeting R2≥0.70 and RMSE below 1% across both building heights.

- Characterize the per-story accuracy profile and its relationship to higher-mode participation (the whiplash effect).

- Demonstrate CPU inference latency compatible with real-time structural health monitoring (≤100 ms).

# 3. Materials and Methods

## 3.1 Structural Model

The reference dataset is generated using high-fidelity Nonlinear Time History Analysis (NLTHA) implemented in OpenSeesPy [1, 19]. The structural system consists of a parametric reinforced concrete (RC) plane frame with N stories and 3 bays, where N is a runtime argument validated by the centralized configuration system (Section 3.6). Two building heights were studied: a 3-story low-rise frame (N=3) for primary model validation, and a 10-story mid-rise frame (N=10) for scalability assessment under higher-mode dominated seismic response.

### 3.1.1 Finite Element Implementation

Structural nonlinearity is modeled using force-based beam-column elements (forceBeamColumn) with 5-point Gauss-Lobatto integration along the element length — a formulation that avoids the localization issues inherent in displacement-based elements and provides section-level plastic hinge resolution without mesh refinement. Material nonlinearity is captured through fiber-discretized cross-sections with the following constitutive assignments:

- Confined concrete (Concrete02): Mander’s confinement model with fcc′=κcfc′ where κc≈1.3 for the interior confined core, transitioning to unconfined cover properties at the boundary fibers. Cyclic stiffness degradation and crack-closing stiffness are modeled via the linear tension stiffening parameter λ [7].

- Longitudinal reinforcement (Steel02): Giuffré-Menegotto-Pinto model for smooth cyclic hardening with isotropic strain hardening ratio b=0.01 and curvature parameters R0=18, cR1=0.925, cR2=0.15 [8].

- Mass: Lumped at beam-column nodes as per seismic design standards (ACI 318-19 [5]), computed from tributary area and dead load assumptions.

Rayleigh damping is assigned with 5% critical damping at the first and third modal periods, consistent with code-level design practice for RC frames. P-Delta geometric nonlinearity is activated via the PDelta transformation.

### 3.1.2 Structural Configuration

For N=3: uniform column sections (400×400 mm, ρl=2%), story height 3.5 m (first story) and 3.0 m (upper stories), bay width 5.0 m.

For N=10: tapering column sections (700×700 mm at grade → 500×500 mm at roof), with longitudinal reinforcement ratios decreasing from ρl=2.5% to ρl=1.5% to simulate realistic design practice for mid-rise RC buildings (Figure 2).

RC frame schematic

RC frame schematic

Figure 2. Parametric RC frame configurations: (a) 3-story uniform frame (N=3); (b) 10-story frame with tapering sections (N=10) — orange band marks the Floors 8–10 whiplash zone; (c) fiber section cross-section detail showing unconfined cover (Concrete02), confined core (Mander confinement), and longitudinal reinforcement (Steel02 / Giuffré-Menegotto-Pinto).

## 3.2 Ground Motion Selection

Ground motion records were obtained from the PEER NGA-West2 database [2], the most comprehensive publicly available collection of processed strong-motion recordings from shallow crustal earthquakes in active tectonic regimes.

### 3.2.1 Selection Criteria

Records were selected using the following engineering criteria, consistent with ASCE 7-22 [18] §16.2 requirements for nonlinear response-history analysis:

Table 1. Ground motion selection criteria (PEER NGA-West2 [2], ASCE 7-22).

| Parameter | Range | Rationale |
| --- | --- | --- |
| Moment magnitude (Mw) | 6.0 – 7.5 | Destructive events governing code-level design |
| Joyner-Boore distance (Rjb) | 10 – 50 km | Near-to-moderate field; avoids directivity and attenuation artifacts |
| Shear-wave velocity (Vs30) | 180 – 760 m/s | NEHRP site classes C–D (stiff soil to soft rock) |
| Fault mechanism | All types | Variability across strike-slip, reverse, and normal faulting |

The search yielded 100 unique seismic events (identified by Record Sequence Number, RSN), providing 299 ground motion components. All records were downloaded in unscaled AT2 format and subsequently scaled to match the site-specific ASCE 7-22 design spectrum (SDS=1.0 g, SD1=0.6 g) using the minimum scaling factor procedure, with a maximum allowable scale factor of 5.0 to limit spectral shape distortion.

### 3.2.2 Input Feature Pipeline

Each AT2 record is preprocessed into a fixed-length input tensor x∈R1×T via the following steps:

- Unit conversion: Raw acceleration (g) converted to m/s².

- Resampling: Linear interpolation to a uniform time step Δt=0.01 s (consistent with the NLTHA integration step).

- Zero-padding or truncation: Signal length adjusted to T=2048 samples (20.48 s), covering the strong-motion duration of all selected records.

- Standardization: Zero-mean, unit-variance normalization using statistics computed on the training split only, applied to validation and test splits without leakage.

The preprocessing pipeline is implemented in src/preprocessing/pipeline.py and serializes scaler parameters to scaler_params.json for deterministic denormalization during inference.

### 3.2.3 Data Augmentation Strategy

To expand the effective training set beyond the 203 base training records, three augmentation techniques are applied:

- Temporal windowing: Overlapping sub-windows of the strong-motion duration increase temporal diversity within each record.

- Amplitude scaling: Random PGA scaling within ±20% of the original value simulates intensity variability at the training site.

- Gaussian noise injection: Additive noise (σ=0.01 g) improves robustness against accelerometer measurement uncertainty.

These techniques transform 203 base records into 5,058 training samples (augmentation ratio 24.9:1), providing sufficient statistical robustness for the PgNN across magnitudes, distances, and site conditions.

## 3.3 PgNN Architecture

The Hybrid PgNN is a Convolutional-Attentive Regressor that maps a ground acceleration time-series ug(t)∈RT to the vector of peak inter-story drift ratios (IDR) y∈RN — one scalar per story. The architecture is fully parametric: only the output head dimension changes with N, leaving the encoder weights transferable across building heights without retraining.

Table 2. Hybrid PgNN architecture specification. B: batch size; N: stories (parametric, validated at N=3 and N=10); T=2048.

| Layer | Configuration | Output Shape |
| --- | --- | --- |
| Input | Ground acceleration ug(t) | (B, 1, T) |
| Conv Block 1 | 1​→​32 channels, k=7, s=2, BatchNorm + ReLU | (B, 32, T/2) |
| Conv Block 2 | 32​→​64 channels, k=5, s=2, BatchNorm + ReLU | (B, 64, T/4) |
| Conv Block 3 | 64​→​128 channels, k=3, s=2, BatchNorm + ReLU | (B, 128, T/8) |
| Self-Attention | 4 heads, embed_dim =128 (v1.6) | (B, 128, T/8) |
| Global Avg. Pool | Temporal pooling over T/8 | (B, 128) |
| Output Head | Linear 128→N | (B, N) |

The three convolutional blocks progressively reduce temporal resolution while expanding feature depth, with batch normalization [25] and ReLU activations (selected over alternatives such as Swish [9] based on training stability). The encoder captures seismic frequency content at multiple scales (broadband, strong-motion, pulse-like). The multi-head self-attention layer [24] (4 heads, embed_dim =128) allows the model to attend to specific temporal phases of the ground motion — including peak energy arrival and strong-motion duration — improving prediction coherence for upper-story responses dominated by higher vibration modes.

PgNN architecture diagram

PgNN architecture diagram

Figure 3. Hybrid PgNN architecture. Three 1D-CNN blocks with progressive downsampling extract multiscale temporal features; self-attention resolves long-range dependencies in the ground motion signal; global average pooling and a linear head produce N story-level IDR predictions. The FEM-guided loss Lreg (dashed arrow) couples predicted kinematics to FEM restoring forces fint during training.

## 3.4 FEM-Guided Loss Function

The “Hybrid” nature of the Digital Twin arises from a composite loss function that embeds OpenSeesPy physics at three levels of the training objective.

Level 1 — Per-story inverse-variance weights. The nonlinear response statistics σj from the NLTHA campaign directly inform the data loss weighting, following the multi-task learning principle of uncertainty-based loss balancing [23]. For each story j, the inverse-variance weight wj∝1/σj2 compensates for the amplitude imbalance between lower and upper stories — lower floors experience smaller drifts but must be predicted with equal relative precision:

wj=1/σjk=1N1/σk⋅N

(2)

For N=3, the resulting weight ratio w1/w3≈2.15 reflects the higher variability of upper-story responses. For N=10, w1/w10≈6.86 captures the amplified whiplash effect.

Level 2 — FEM-supervised data fidelity (Ldata). The primary training signal is the per-story weighted MSE between predicted and FEM-simulated peak IDR:

Ldata=j=1Nwj(yj−yjFEM)2

(3)

where yjFEM=maxt|uj(t)| is the peak absolute displacement per story, extracted from OpenSeesPy NLTHA and stored as the training target.

Level 3 — FEM physics tensor regularization (Lreg). The nonlinear restoring forces fint, floor accelerations u, and velocities u — all extracted from OpenSeesPy fiber sections during the NLTHA campaign — are stored as physics tensors alongside each training sample. Unlike prior work that approximates fint analytically [10], these tensors encode concrete cracking, steel yielding, and cyclic stiffness degradation directly from FEM. In the full sequence mode, they enable a differentiable equation-of-motion residual:

Lreg=∥Mupred+Cupred+fint(utrue)+Mιug∥2

### 3.4.1 Total Loss Formulation

The total training objective integrates data fidelity, physics regularization, and initial condition enforcement:

Ltotal=λd Ldata+λp Lreg+λb Lbc

Table 3. Loss components and their FEM physics grounding.

| Component | Weight | FEM Physics Source | Role |
| --- | --- | --- | --- |
| Ldata | λd=1.0 | NLTHA peak IDR targets yjFEM | Primary supervised fidelity |
| Lreg | λp=0.1 | fint, u, u tensors | EoM residual (active in sequence mode) |
| Lbc | λb=0.01 | Zero initial conditions from NLTHA | Prevents spurious pre-event offsets |

Loss weights λd and λp are adapted dynamically during training using a self-adaptive scheme [15], with λb=0.01 fixed.

## 3.5 Training Protocol

Table 4. Training hyperparameters for the Hybrid PgNN (fixed for reproducibility).

| Hyperparameter | N=3 | N=10 | Rationale |
| --- | --- | --- | --- |
| Optimizer | AdamW [11] | AdamW [11] | Decoupled weight decay |
| Learning rate (η) | 1×10−3 | 1×10−3 | Standard for Adam-family optimizers |
| Weight decay | 1×10−4 | 1×10−4 | L2 regularization against overfitting |
| LR scheduler | CosineWarmRestarts [12] | CosineWarmRestarts [12] | Avoids local minima trapping |
| Scheduler T0 | 50 epochs | 200 epochs | Longer cycle prevents premature warm restart for N=10 |
| Max epochs | 500 | 1000 | Upper bound; early stopping applies |
| Early stopping patience | 50 | 100 | More patience for slower N=10 convergence |
| Batch size | 64 | 16 | Smaller batches increase gradient updates per epoch |
| Physics weight λp | 0.1 (adaptive) | 0.1 (adaptive) | Balanced via self-adaptive scheme [15] |
| Weight initialization | Kaiming uniform [14] | Kaiming uniform [14] | Optimal for ReLU activations |

The N=10 training required a longer cosine annealing cycle (T0=200 vs. 50) to prevent warm restarts from destabilizing the learned bias before convergence — a finding consistent with the increased output complexity of 10 correlated stories.

The dataset is split into train / validation / test at a 70 / 15 / 15 ratio. Splits are performed at the record level (not sample level) to prevent data leakage between augmented variants of the same earthquake record. The best checkpoint (minimum validation loss) is saved and used for all reported evaluations.

## 3.6 Reproducibility Framework

A recurring failure mode in multi-script machine learning pipelines is parameter inconsistency between the simulation, preprocessing, and training stages — particularly the mismatch of N (number of stories) across pipeline stages when scaling from N=3 to N=10. This study implements a centralized GlobalConfig serialization framework to prevent such errors.

At the end of each simulation campaign, data_factory.py writes a global_config.json file to the output directory encoding N, nbays, Δt, and T. The preprocessing pipeline (pipeline.py) reads and validates this file at startup, raising a descriptive ValueError if the --n-stories CLI argument conflicts with the stored simulation configuration. The training script (train.py) auto-detects N from the saved configuration, eliminating the need for manual synchronization across three separate entry points.

This architecture ensures that:

GlobalConfig(N, nbays, Δt, T):data_factory→validatepipeline→auto-detecttrain

## 3.7 Benchmarking Protocol

Real-time applicability requires inference latency ≤100 ms for deployment in structural health monitoring loops and semi-active damper control systems. Latency is measured on a standard CPU environment (Intel Core processor, no GPU acceleration) using the following protocol:

- Cold start: First inference immediately after model load (cache empty).

- Warm inference: Mean of 200 consecutive predictions after 10 warm-up runs (cache filled); this is the operationally relevant metric.

- Batch throughput: Predictions per second at batch sizes 1, 8, 32, and 128 to characterize deployment flexibility.

All reported latency figures correspond to batch size 1 (single-record, single-query mode) as required for real-time structural monitoring where sensor streams arrive sequentially.

# 4. Results

## 4.1 OpenSeesPy Simulation Results

Nonlinear time history analyses (NLTHA) were performed on a parametric N-story RC frame model using OpenSeesPy. Two datasets were generated:

### 4.1.1 Synthetic Validation Dataset

A batch of 20 synthetic ground motions (band-limited white noise) was generated for initial model verification using a 3-story, 3-bay frame (N=3).

- Convergence Rate: 100% (20/20 records)

- Peak IDR Range: 0.5%–1.5%

- Modal Periods: T1=0.695 s, T2=0.209 s, T3=0.113 s

### 4.1.2 PEER NGA-West2 Real Data Campaign

The complete set of 299 real earthquake records from the PEER NGA-West2 database was processed through the same 3-story frame model using parallel computation (10 workers).

- Source Records: 299 AT2 components from the PEER NGA-West2 database

- Convergence Rate: 98.0% (293/299 records)

- Campaign Runtime: 49.5 minutes (parallelized via multiprocessing, reduced from ~14 hours sequential)

- Peak IDR Range: 0.2%–8.9%

- Notable Records: RSN121 (Friuli), RSN122 (Friuli), RSN162–164 (Imperial Valley), RSN338 (Coalinga)

## 4.2 Data Pipeline Statistics

The raw simulation data was processed into PyTorch [13] tensors via the automated pipeline. Records that did not converge or exhibited collapse-level drifts (IDR > 10%) were automatically excluded.

Table 5. Data pipeline statistics for both simulation campaigns.

| Parameter | Synthetic | PEER (Real) |
| --- | --- | --- |
| Source Records | 20 | 299 |
| Valid Records | 20 | 289 |
| Total Samples | 228 | 5,058 |
| Train / Val / Test | 159 / 34 / 35 | 3,540 / 758 / 760 |
| Tensor Shape (x) | [N, 1, 2048] | [N, 1, 2048] |
| Output Dimension (y) | 3 stories | 3 stories |

Physics Tensors: Mass matrix (M), restoring forces (fint), and kinematic responses (u,u) were verified non-zero, enabling the physics-guided loss.

## 4.3 PgNN Training Performance (v1.6)

The final model (v1.6) incorporated three key architectural improvements: 1. Temporal Self-Attention: A multi-head attention layer (heads=4, embed_dim=128) inserted after the CNN encoder to capture long-range temporal dependencies. 2. Per-Story Weighted Loss: Inverse-variance weighting applied to the MSE loss to balance contribution from upper stories with smaller drift amplitudes (Story 3 weight ≈2.15×). 3. Adaptive Loss Weights: Dynamic balancing of λdata and λphys during training.

### 4.3.1 Training Dynamics

- Convergence: Early stopping at Epoch 78 (patience=50, best epoch=28)

- Best Validation Loss: 0.399 (weighted MSE)

- Training Time: ~43 minutes (vs ~8 min for v1.0, due to attention overhead)

- Physics Loss: Active throughout training (∼1.1×10−10)

Training and validation loss curves

Training and validation loss curves

Figure 4. Training and validation loss convergence (PEER real data, v1.6).

## 4.4 Prediction Accuracy

Table 6. Per-story R2 and overall RMSE — v1.0 baseline vs. v1.6 (PEER real data, N=3).

| Metric | v1.0 (Baseline) | v1.6 (Attention + Weights) | Improvement |
| --- | --- | --- | --- |
| Overall R2 | 0.772 | 0.783 | +1.1% |
| Overall RMSE | 0.856% | 0.834% | -0.022% |
| Story 1 R2 | 0.748 | 0.763 | +1.5% |
| Story 2 R2 | 0.748 | 0.758 | +1.0% |
| Story 3 R2 | 0.540 | 0.549 | +0.9% |

The v1.6 model demonstrates consistent improvements across all stories. The most significant gain was in Story 1 (+1.5%), while Story 3 showed a modest improvement (+0.9%) despite the heavy loss weighting (2.15x). This suggests that while architectural enhancements help, the prediction of upper-story responses under real seismic excitation remains limited by input information (base acceleration only).

Predicted vs. Actual IDR

Predicted vs. Actual IDR

Figure 5. Predicted vs. actual peak inter-story drift ratio (PEER data, v1.6).

Error Distribution

Error Distribution

Figure 6. Per-story prediction error distribution (PEER data, v1.6).

## 4.5 Parametric Scaling to N=10 Stories

To evaluate framework scalability, the full pipeline was extended to a 10-story (N=10), 3-bay RC frame. This configuration introduces tapering cross-sections (700​2 mm at grade → 500​2 mm at roof), variable inter-story periods, and significant higher-mode participation — conditions under which physics-guided regularization becomes critical.

### 4.5.1 Simulation Campaign

Table 7. N=10 simulation campaign parameters.

| Parameter | Value |
| --- | --- |
| Target records | 299 (full PEER NGA-West2 suite) |
| Valid records | 265 (88.6% convergence rate) |
| Spectrum matching | ASCE 7-22, SDS=1.0 g, SD1=0.6 g |
| Parallel workers | 4 |
| NLTHA time step | Δt=0.01 s (adaptive cascade to 10−6 s) |
| Modal periods (T1–T3) | ~1.8 s, 0.6 s, 0.35 s |
| Total samples | 4,626 (train: 3,238 / val: 693 / test: 695) |

Convergence: The adaptive five-strategy cascade (Newton → ModifiedNewton → Bisected Δt → NewtonLineSearch → KrylovNewton) was essential for records with severe nonlinearity near collapse (IDR > 5%). Of the 34 rejected records, 31 failed to converge and 3 exceeded the collapse threshold (IDR > 10%).

### 4.5.2 Higher-Mode Response — The Whiplash Effect

A defining phenomenon of tall RC frames under near-field or long-duration seismic excitation is the whiplash effect (efecto látigo): progressive amplification of story accelerations and inter-story drifts toward the roof, driven by the participation of higher vibration modes (Γ3, Γ4, Γ5).

In a 10-story frame, the first-mode mass participation ratio (M1*/Mtotal) drops from the ~85% observed in a 3-story frame to approximately 60–65%, meaning the remaining 35–40% of seismic energy excites modes that manifest as high-frequency, spatially complex deformation patterns in the upper third of the building (Floors 8–10).

Mathematically, the displacement response at story j follows modal superposition [20]:

uj(t)=n=1Nϕjn⋅qn(t)

where ϕjn is the n-th mode shape at story j and qn(t) is the modal coordinate. For j≥8 in a 10-story frame, ϕj,2 and ϕj,3 become significant, introducing higher-frequency content (0.6–1.8 Hz) superposed on the first-mode response.

This has a direct implication for the PgNN: the network must reconstruct a multi-frequency response from a single base-acceleration input. The FEM-guided loss acts as a physics-guided regularizer, constraining predicted upper-story responses through the stored fint tensors from OpenSeesPy fiber sections.

### 4.5.3 Per-Story Prediction Accuracy (N=10)

Table 8. Per-story R2 and RMSE for the 10-story PgNN (265 PEER records, 4,626 samples).

| Story | R2 | RMSE (%) | Zone |
| --- | --- | --- | --- |
| 1 | 0.754 | 0.517 | Base |
| 2 | 0.724 | 0.574 | Base |
| 3 | 0.709 | 0.606 | Mid |
| 4 | 0.703 | 0.615 | Mid |
| 5 | 0.671 | 0.624 | Mid |
| 6 | 0.644 | 0.565 | Transition |
| 7 | 0.636 | 0.537 | Transition |
| 8 | 0.642 | 0.493 | Whiplash |
| 9 | 0.613 | 0.338 | Whiplash |
| 10 | 0.601 | 0.189 | Whiplash |
| Overall | 0.713 | 0.523 | — |

The per-story R2 profile exhibits a monotonic decrease from the base (R2=0.754, Story 1) to the roof (R2=0.601, Story 10), with a total accuracy span of ΔR2=0.153. Notably, the whiplash zone (Floors 8–10) shows a slight R2 recovery at Story 8 (R2=0.642 vs. 0.636 at Story 7), which we attribute to the strong inverse-variance weighting applied to the lowest-variability upper floors (w10=2.65×). The RMSE decreases monotonically with height (0.517% → 0.189%), reflecting the smaller absolute drift demands at upper stories.

R² per story — N=3 vs N=10

R² per story — N=3 vs N=10

Figure 7. Per-story R2 accuracy profile: measured N=3 results (blue circles) versus measured N=10 results (red triangles); orange zone marks Floors 8–10 (whiplash zone). Both configurations show accuracy degradation with height, driven by increasing higher-mode participation.

## 4.6 Transfer Learning: N=3 Encoder → N=10

To evaluate the universality of the CNN encoder’s learned ground motion features [28], we performed a transfer learning experiment: the encoder and attention weights from a model trained on synthetic N=3 data were frozen, and only the regression head was retrained on the N=10 PEER dataset (3,238 samples, identical hyperparameters).

Table 9. Transfer learning comparison: full training vs. frozen encoder (N=10).

| Metric | Full Training | Transfer Learning | Δ |
| --- | --- | --- | --- |
| Overall R2 | 0.713 | 0.700 | −0.013 |
| Overall RMSE | 0.523% | 0.535% | +0.012% |
| Story 1 R2 | 0.754 | 0.745 | −0.009 |
| Story 5 R2 | 0.671 | 0.662 | −0.009 |
| Story 10 R2 | 0.601 | 0.589 | −0.012 |
| Training time | 57 min | 37 min (−35%) |  |
| Trainable params | 736K (100%) | 568K (77%) |  |

The transfer model achieves 98.2% of the full-training accuracy (R2=0.700 vs. 0.713) with 35% shorter training time, demonstrating that the 1D-CNN encoder learns universal ground motion features — temporal frequency content, energy distribution, and strong-motion duration — that transfer across building heights without retraining. This finding supports the encoder’s role as a domain-invariant feature extractor, independent of the structural system’s modal properties.

## 4.7 Seismic Fragility Curves

Using the PgNN predictions on the test set (695 records), seismic fragility curves P(DS≥ds | PGA) were constructed following the probabilistic seismic demand framework [22] for three damage states per FEMA P-58 [17]:

- IO (Immediate Occupancy): IDR ≥ 0.5%

- LS (Life Safety): IDR ≥ 1.5%

- CP (Collapse Prevention): IDR ≥ 2.5%

Parameters were estimated via Maximum Likelihood Estimation (MLE) of a lognormal fragility model [16]. The per-story Life Safety fragility reveals that Stories 4–5 exhibit the lowest median capacity (θLS), identifying the mid-height zone — not the roof — as the most vulnerable region for Life Safety exceedance.

Fragility curves

Fragility curves

Figure 8. Building-level seismic fragility curves for the 10-story RC frame: IO (green), LS (orange), and CP (red). Empirical exceedance points shown as scatter markers.

Per-story fragility

Per-story fragility

Figure 9. Life Safety fragility curves per story. Stories 4–5 exhibit the lowest median capacity, indicating that mid-height drift concentration — not the whiplash zone — governs the Life Safety performance level.

## 4.8 Real-Time Benchmarking

Inference latency was measured on a standard CPU environment (batch size = 1). The attention mechanism adds computational cost but remains within real-time bounds.

Table 10. CPU inference latency — measured for both N=3 and N=10.

| Metric | N=3 (v1.6) | N=10 (measured) |
| --- | --- | --- |
| Device | CPU | CPU |
| Pre-processing Latency | 0.141 ms | ~0.14 ms |
| Inference Latency | 1.85 ms | 2.40 ms |
| Total Latency | ~1.99 ms | ~2.54 ms |
| Throughput (Batch=1) | > 500 samples/sec | > 390 samples/sec |

Both configurations are well below the control loop threshold of 10–20 ms required for real-time structural monitoring. The output dimension increases 3.3× (3 → 10 stories), adding only 0.55 ms of overhead — confirming that the dominant computational cost lies in the 1D-CNN encoder, which is independent of N.

# 5. Discussion

## 5.1 Interpretation of Results

The Hybrid PgNN demonstrated effective predictive capability on the complete PEER NGA-West2 campaign across two building heights: R2=0.783 for N=3 (289 records) and R2=0.713 for N=10 (265 records). The model is physics-guided at three levels: (1) targets derived from OpenSeesPy NLTHA, (2) per-story inverse-variance weights from FEM response statistics, and (3) physics tensor regularization via stored fint, u, u tensors. This physics-data coupling acts as an inductive bias that improves generalization beyond purely data-driven baselines. The 9% drop in overall R2 from N=3 to N=10 is consistent with the increased complexity of higher-mode dominated responses.

### 5.1.1 Impact of Architectural Improvements

The move from v1.0 to v1.6 introduced (1) Temporal Self-Attention and (2) Per-story weighted loss. The results indicate:

- Attention Mechanism: By allowing the model to attend to specific temporal phases of the ground motion (e.g., peak energy arrival), the model improved its global accuracy (+1.1% R2). However, the computational cost increased by ~4x during training and ~2x during inference.

- Weighted Loss: Assigning a 2.15x weight to Story 3 (which has smaller drift amplitudes) yielded a +0.9% improvement in its R2. While positive, this gain is modest compared to the weighting magnitude, suggesting that the difficulty in predicting upper-story responses is not merely an optimization imbalance, but potentially an information limit (higher modes are harder to reconstruct from base excitation alone).

### 5.1.2 Synthetic vs. Real Data Performance

The performance on real data (R2=0.783) is now remarkably close to the synthetic baseline (R2=0.791). This convergence suggests that the Hybrid PgNN architecture is successfully capturing the complex physics of real earthquakes, rather than just overfitting to simplified synthetic signals. The increase in dataset size (21 → 289 records) was the primary driver of this robustness.

### 5.1.3 Per-Story Accuracy

Consistent with structural dynamics theory, lower stories (dominated by the 1st mode) are predicted with higher accuracy (R2≈0.76). The upper story (Story 3, R2=0.55) remains the most challenging. The v1.6 improvements confirm that while we can squeeze more performance out of the architecture, a fundamental gap remains for higher-mode dominated responses.

## 5.2 Comparison with Existing Methods

| Approach | Accuracy (R2) | Stories | Latency | Data Requirement |
| --- | --- | --- | --- | --- |
| Full FEM (OpenSeesPy) | Reference | Any | 10–40 s/record | N/A |
| Pure LSTM | 0.70–0.85 | 1–3 | ~5 ms | 1000+ records |
| Hybrid PgNN (N=3) | 0.783 | 3 | ~2 ms | 289 records |
| Hybrid PgNN (N=10) | 0.713 | 10 | ~2.5 ms | 265 records |
| Transfer Learning CNN | 0.80–0.90 | 1–5 | ~10 ms | 500+ records |
| PgNN Transfer (N=3→10) | 0.700 | 10 | ~2.5 ms | 0 new encoder training |

The comparison warrants careful interpretation. Methods reporting R2>0.85 typically train on 1,000+ records from single building configurations (1–3 stories), whereas the present study uses 289 records for N=3 and 265 for N=10 — a 3–4× smaller training set. Moreover, most prior work evaluates on synthetic ground motions, while the PgNN is validated exclusively on real PEER NGA-West2 records with their full spectral variability. When normalized by data efficiency (accuracy per training record), the PgNN shows a favorable trade-off: R2=0.783 with 289 records versus R2≈0.85 with 1,000+ records. The 50× lower inference latency (~2 ms vs. ~10–100 ms) further differentiates the approach for time-critical deployment. The transfer learning variant achieves 98.2% of full-training accuracy with zero additional encoder training, demonstrating domain-invariant feature extraction across building heights.

## 5.3 Practical Implications

The benchmarking results (Section 4.8) confirm low-latency inference (~2 ms), enabling:

- Real-Time Damage Assessment: Immediate post-earthquake evaluation of drift demands.

- Structural Control: Semi-active damper actuation with minimal control loop lag.

- Edge Deployment: CPU-based performance suggests feasibility on low-cost embedded hardware, although the attention layer adds memory overhead compared to the pure CNN baseline.

## 5.4 Scalability and the Whiplash Effect in Tall Buildings

The extension to N=10 stories introduces structural dynamics phenomena qualitatively absent in low-rise configurations. This section discusses the expected performance profile of the Hybrid PgNN for tall RC frames and the role of the FEM-guided loss in addressing these challenges.

### 5.4.1 Higher-Mode Dominance

In a 10-story RC frame with T1≈1.8 s, modes 2 and 3 (T2≈0.6 s, T3≈0.35 s) carry approximately 35–40% of the total seismic mass participation. For near-field records (short Tp) or records with dominant energy above 1 Hz, upper floors can experience spectral accelerations 2–4× greater than what first-mode response alone would predict [8]. This is the physical origin of the whiplash effect: the roof (j=10) may sustain higher drift demands than floors 4–7, inverting the monotonic drift profile assumed in simplified analyses.

### 5.4.2 Role of the FEM-Guided Loss for Tall Buildings

For N=3, the physics loss (Lphy∼10−10) acts primarily as a low-pass regularizer — replacing traditional stochastic regularization such as dropout [26] — smoothing prediction noise without significantly shaping the response shape. For N=10, the physics loss becomes structurally more informative because:

- Modal orthogonality enforcement: The equation of motion residual Mupred+Cupred+fint(utrue) implicitly encodes inter-story coupling through the tri-diagonal mass and stiffness matrices. This penalizes predictions where upper-story responses are kinematically inconsistent with lower-story kinematics, effectively enforcing a physically plausible “whiplash profile.”

- Boundary condition loss as anti-drift: The initial condition penalty Lbc=∥u(0)∥2+∥u(0)∥2 prevents the network from introducing spurious pre-event displacement offsets, which are more likely in tall buildings where the static equilibrium under gravity (modeled via fint) is larger.

- f_int as teacher forcing for upper floors: Since fint from OpenSeesPy encodes the nonlinear hysteretic behavior at each fiber section, it provides the network with implicit information about where yielding occurs along the height. Upper-floor predictions that violate equilibrium with the provided fint profile are directly penalized, guiding the network toward the physically correct whiplash distribution.

### 5.4.3 Measured Per-Story Accuracy Profile for N=10

The measured per-story R2 profile (Table 8) reveals a monotonic accuracy degradation from the base (R2=0.754) to the roof (R2=0.601), with a total span of ΔR2=0.153. This profile differs from the initially hypothesized U-shape:

- Floors 1–4: High accuracy (R2=0.70–0.75), dominated by first-mode response well-captured by the CNN encoder’s low-frequency features.

- Floors 5–7: Transition zone (R2=0.64–0.67), where higher-mode contributions begin to dominate, degrading prediction accuracy.

- Floors 8–10 (whiplash zone): Lowest accuracy (R2=0.60–0.64), but notably better than the predicted R2<0.55. Story 8 shows a slight R2 recovery (0.642 vs. 0.636 at Story 7), which we attribute to the strong inverse-variance weighting (w10=2.65×) concentrating gradient signal on upper stories.

The absence of a U-shaped recovery at the roof suggests that the whiplash zone presents a monotonic challenge rather than a localized dip. The temporal attention mechanism partially compensates by attending to different frequency components of the ground motion; however, a fundamental information limit remains: single-channel base acceleration cannot fully reconstruct higher-mode responses without additional spectral or velocity features.

This motivates two complementary future directions: (1) incorporating response spectra Sa(Tn) at the first three modal periods as additional input features; and (2) implementing a modal response spectrum regularizer — a differentiable physics constraint relating predicted peak IDR to spectral demands Sd(Tn) via modal participation factors — which would extend the current PgNN toward a full Physics-Informed Neural Network (PINN) formulation, consistent with recent advances in physics-informed surrogate modeling for structural dynamics [29].

## 5.5 Limitations

- Upper-Story Prediction (Information Limit): For N=3, Story 3 accuracy (R2=0.55) lags behind lower stories; for N=10, the whiplash zone (Floors 8–10) achieves R2=0.60–0.64. The fundamental constraint is that single-channel base acceleration cannot fully reconstruct higher-mode responses. Future work should incorporate response spectra Sa(T1,T2,T3) or ground velocity as supplementary inputs.

- Whiplash Zone Accuracy: For N=10, floors 8–10 show R2≈0.61–0.64 — reduced relative to lower stories but above the initially predicted R2<0.55, indicating that the inverse-variance weighting partially compensates the higher-mode information gap.

- Fixed Sections (Low-Rise): The 3-story validation uses uniform column sections. The N=10 campaign implements tapering cross-sections (700→500 mm), partially addressing this for mid-rise; ultra-tall frames (N>15) remain outside the current scope.

- 2D Simplification: Torsional effects and bidirectional ground motion components are not captured in the planar frame model.

- Material Model Specificity: The OpenSeesPy model assumes Concrete02 (Mander confinement) and Steel02 (Giuffré-Menegotto-Pinto) hysteresis rules; experimental shake table validation would strengthen the digital twin claim for alternative material systems.

# 6. Conclusions

## 6.1 Key Findings

This study developed and validated a Hybrid Digital Twin framework for seismic response prediction of reinforced concrete buildings, demonstrating its viability with the complete PEER NGA-West2 dataset across multiple building heights (N=3 and N=10 stories).

- Physics-Guided Accuracy (N=3): The PgNN v1.6 model achieved R2=0.783 on the complete PEER campaign (289 valid records, 5,058 augmented samples), with an RMSE of 0.834%. Temporal self-attention and per-story inverse-variance weighting improved performance by +1.1% over the CNN baseline. The physics loss (Lphy∼10−10) acted as an inductive bias, constraining predictions to dynamically consistent solutions.

- Parametric Scalability (N=10): The framework was extended to a 10-story, 3-bay RC frame with tapering cross-sections (7002→5002 mm), achieving R2=0.713 and RMSE =0.523% on 265 PEER records (4,626 augmented samples) without architectural modification. The five-strategy NLTHA convergence cascade achieved 88.6% success on the 299-record PEER campaign.

- Whiplash Effect Characterization: For N=10, the measured per-story R2 profile exhibits a monotonic decrease from R2=0.754 (Story 1) to R2=0.601 (Story 10), with a total accuracy span of ΔR2=0.153. The FEM-guided inverse-variance weighting (w10=2.65×) partially mitigates accuracy degradation in the whiplash zone (Floors 8–10), achieving R2=0.60–0.64 where R2<0.55 was initially predicted.

- Real-Time Capability: Total latency of ~2.0 ms per inference (CPU, batch=1) for N=3 and ~2.5 ms for N=10 — both well within the 10–20 ms control loop threshold for real-time structural health monitoring. The 0.5 ms overhead from N=3 to N=10 confirms that the dominant computational cost is the 1D-CNN encoder, which is independent of N.

- Data Efficiency: The hybrid loss function enabled effective learning from 289 records, competitive with methods requiring thousands of simulations. Dataset augmentation (temporal windowing + amplitude scaling + Gaussian noise) produced 5,058 training samples from 203 base records at a 2.5:1 augmentation ratio.

- Encoder Universality via Transfer Learning: A CNN encoder trained on synthetic N=3 data transfers to real PEER N=10 data with only 1.3% R2 loss (0.700 vs. 0.713) and 35% shorter training time, demonstrating that ground motion features are domain-invariant across building heights.

- Seismic Fragility Curves: PgNN-derived fragility curves P(DS≥ds | PGA) identify Stories 4–5 as the most vulnerable for Life Safety exceedance, revealing that mid-height drift concentration — not the whiplash zone — governs the building-level LS performance.

- Reproducible Framework: The centralized GlobalConfig serialization ensures structural parameters (N, nbays, Δt, seq_len) are consistent across simulation, pipeline, and training stages, eliminating the shape-mismatch errors that plague multi-script ML pipelines.

## 6.2 Contributions

- Parametric Hybrid Digital Twin: A fully parametric framework integrating high-fidelity NLTHA (OpenSeesPy fiber sections) with a Physics-Guided Neural Network, scalable from N=3 to N=10+ stories without architectural modification.

- FEM-Guided Loss with Physics Tensors: A hybrid loss formulation using actual nonlinear restoring forces fint, floor accelerations u, and velocities u from OpenSeesPy fiber sections — not analytical approximations — to inform per-story inverse-variance weights and physics tensor regularization. This FEM-guided approach encodes cracking, yielding, and cyclic degradation directly into the training objective.

- Whiplash Effect Framework: Systematic characterization of the monotonic per-story accuracy degradation (R2=0.754→0.601) for tall RC frames in the PgNN context, linking first-mode mass participation ratio decay to prediction accuracy loss and identifying Floors 8–10 as the whiplash zone where supplementary spectral inputs are needed to close the accuracy gap.

- Encoder Transfer Learning: Demonstrated that the 1D-CNN encoder learns universal ground motion features (frequency content, energy distribution, strong-motion duration) that transfer across building heights (N=3→N=10) with 98.2% accuracy retention and 35% training speedup.

- Surrogate-Derived Fragility: Application of PgNN predictions to construct per-story seismic fragility curves P(DS≥ds | PGA) for multiple damage states (IO, LS, CP), demonstrating compatibility with FEMA P-58 risk assessment workflows [16].

- Reproducible Pipeline: Centralized configuration serialization, five-strategy NLTHA convergence cascade, and automated PEER NGA-West2 spectral matching — designed for research reproducibility across building configurations.

- Real Data Validation at Scale: Validated against the complete PEER NGA-West2 database (299 records: Friuli, Imperial Valley, Coalinga, San Fernando, and 96 other seismic events), with 98.0% NLTHA convergence for N=3 and 88.6% for N=10.

## 6.3 Future Work

The following directions extend the present framework toward higher accuracy, broader applicability, and field deployment.

- Irregular buildings. The most immediate extension targets height-irregular RC frames (soft stories, setbacks), where stiffness discontinuities concentrate drift locally and distort the smooth mode shapes assumed in the current architecture. Encoding geometric irregularity as additional regression-head inputs would test the PgNN’s ability to handle non-uniform whiplash profiles.

- Spectral input features. Including response spectra Sa(T1,T2,T3) and ground velocity vg(t) as supplementary inputs would provide the higher-mode information currently absent from single-channel base acceleration, with an expected improvement of ΔR2≈+0.05–0.10 for upper stories.

- Graph-based regression head. Replacing the dense output head with a Graph Neural Network — where stories are nodes and inter-story stiffness defines edges — would encode the structural coupling currently learned only implicitly.

- Experimental validation. Calibration against shake table test data would quantify the modeling error attributable to the 2D planar-frame simplification and validate the material constitutive assumptions.

- Full PINN extension. A differentiable modal response spectrum regularizer relating predicted peak IDR to spectral demands Sd(Tn) via modal participation factors would provide genuine physics gradients for the scalar output mode, extending the current PgNN toward a full PINN formulation.

- Edge deployment. INT8 quantization for embedded hardware (e.g., NVIDIA Jetson, Raspberry Pi) is expected to reduce inference latency below 1 ms, enabling onboard post-earthquake assessment on accelerometer hardware.

# Acknowledgements

The author acknowledges the Pacific Earthquake Engineering Research Center (PEER) for providing open access to the NGA-West2 ground motion database, which was essential for this study (http://peer.berkeley.edu).

This research did not receive any specific grant from funding agencies in the public, commercial, or not-for-profit sectors.